University at Buffalo

Checklist for Safe Use of Biological Safety Cabinets

Preparing for Work in a BSC:

- Put on PPE.

- Turn UV light OFF (if used).

- Turn fluorescent light ON.

- Turn the cabinet ON, allow it to run for 4 minutes (or Manufacturer’s recommended time) to purge the BSC of particulates. Some cabinets may alarm until purge is complete.

- Verify proper sash height and that sash alarm is ON.

- Verify drain valve underneath the cabinet is closed (valve handle is perpendicular to valve body).

- Check cabinet’s certiﬁcation sticker expiration date is within 1 year.

- Check the pressure differential gauge reading (if present), compare it against the calibration set point.

- Decontaminate all surfaces of the cabinet.

- Collect all materials needed for the work.

Use of a BSC:

- Place absorbent plastic-backed material to protect the work surface, if required.

- Decontaminate the external surfaces of any equipment or supplies that you will need to place in the BSC.

- Place materials as far back in the cabinet as practical without blocking front and rear grilles.

- Separate clean/sterile items from dirty/potential contaminated items inside the BSC.

- Work from clean to dirty to minimize cross-contamination.

- One person at a time should work in a cabinet.

- Move your arms slowly in and out of the BSC. Do not move arms in sweeping, sideways movements.

- Do not use open flame.

- Adjust the stool/bench height so your face is above the bottom of the sash and your arms enter the cabinet with elbows at 90° angles, armpits level with the bottom of the sash. Use a foot rest if your feet do not touch the floor.

- Discard all waste in a biohazard bag inside of the BSC.

After Completing Work in a BSC:

- Do NOT turn cabinet OFF while removing items and decontaminating the cabinet.

- Close and surface decontaminate ALL containers and lab materials before removal.

- Surface decontaminate the exterior of the biohazard waste bag/container before removal and disposal.

- Decontaminate cabinet interior, including sidewalls, back wall, inside of sash and work surface.

- Remove the gloves you were using in the BSC and dispose of them as Regulated Medical Waste.

- Turn fluorescent light and blower motor switches OFF.

- Turn UV light ON. Keep the sash closed while the UV light is on.

- Wash your hands.

Questions?

Environment, Health and Safety, 220 Winspear Avenue. Buffalo, NY 14215

716.829-3301   ehs@facilities.buffalo.edu

buffalo.edu/ehs